Virginia Graeme Baker Pool & Spa Safety Act

CPSC Staff’s Guide to Complying with the Law

By December 19, 2008, ALL public pool owners/operators must:

- Install drain covers that meet the ANSI/ASME A112.19.8-2007 standard on EVERY drain/grate.  A list of cover manufacturers can be found at www.cpsc.gov/whatsnew.html#pool.

- Install an automatic shut-off system, gravity drainage system, Safety Vacuum Release System (SVRS),  or suction-limiting vent system.  Pools and spas with unblockable drains are exempt from this requirement.  A list of SVRS manufacturers can be found at www.cpsc.gov/whatsnew.html#pool.

- Ensure that dual or multiple main drains are at least 3 feet apart.

Pools and spas not open on December 19, 2008, are not required to be in compliance until the day the pool or spa is reopened!

The new Consumer Product Safety Improvement Act requires all new drain and grate covers be certified to applicable standards starting on November 12, 2008.

Have a question or concern about the Pool and Spa Safety Act?

Send an email to info@cpsc.gov.